GoldenEye

Prepared By:

Hamza Jad Allah - 22110444

Suhaib Alajami - 22110069

Omar Malkawi - 22110244

Supervised By:

Dr. Rami Al-Ouran

Project Submitted in partial fulfillment for the degree of Bachelor of Science in Data Science & Artificial Intelligence

Fall-2025

Table of Contents

1.1 Background	4

1.2 Motivation	5

1.2.1 The Humanitarian Crisis: The Sensitive Few Hours in the Empty Quarter	6

1.2.2 The Economic Driver: Cost-Asymmetry of Rescue	6

1.2.3 Strategic Alignment: Supporting Vision 2030 & Regional Growth	6

1.3 Problem Statement	8

1.3.1 Project Objectives	9

Main Requirements	9

1.3.2 User Requirements	10

The primary users of the proposed system are search and rescue (SAR) operators and emergency response teams conducting missions in desert and remote environments. The system must support their operational needs by providing timely, reliable, and actionable information while minimizing risk and resource usage.	10

Functional Requirements	10

1.	Human Detection Capability	10

	The system shall be able to detect humans from aerial imagery captured by a UAV.	10

	Detection shall support top-down and high-oblique viewing angles typical of drone flights.	10

	The system shall provide bounding boxes and confidence scores for detected humans.	10

2.	Real-Time Operation	10

	The system shall perform inference in real time or near real time to support time-critical SAR missions.	10

	Detection results shall be generated continuously from a live or simulated drone video stream.	10

3.	Desert Environment Robustness	10

	The system shall operate reliably in desert-specific conditions, including:	10

1.	Low visual contrast between humans and background	10

2.	Harsh and uneven lighting	10

3.	Motion blur due to drone movement	10

4.	Partial occlusion and small target sizes	10

4.	Autonomous Processing	10

	The system shall operate with minimal user intervention once deployed.	10

	No manual image selection or annotation shall be required during inference.	10

5.	Visual Feedback for Operators	10

	The system shall display detection results visually, overlaying bounding boxes on the video feed.	10

	Confidence levels shall be presented in an interpretable manner to assist decision-making.	10

Non-Functional Requirements	11

1.	Accuracy and Reliability	11

	The system shall achieve high detection accuracy, targeting a mean Average Precision (mAP) greater than 95% for relevant classes.	11

	False negatives shall be minimized, as missed detections directly impact mission success.	11

2.	Edge Deployment Compatibility	11

	The system shall be deployable on a Raspberry Pi 5 or equivalent edge device.	11

	The solution shall function within the computational and memory constraints of the platform.	11

3.	Performance Efficiency	11

	The system shall maintain acceptable inference speed (frames per second) suitable for live aerial monitoring.	11

	Resource usage (CPU, memory, and temperature) shall remain within safe operational limits.	11

4.	System Stability	11

	The system shall run continuously for extended periods without crashing or significant performance degradation.	11

	The system shall recover gracefully from temporary input interruptions (e.g., dropped frames).	11

5.	Portability and Ease of Deployment	11

	The system shall be easy to set up and deploy in simulated or real SAR environments.	11

	Configuration and execution shall be manageable by users with basic technical training.	11

1.3.4 Operational Constraints	12

1.3.5 Relevant Computing Characteristics	12

Real-Time Processing	12

Edge Computing Constraints	12

High-Throughput Visual Data Processing	12

Robustness to Noisy and Degraded Inputs	13

System Reliability and Fault Tolerance	13

1.4 Related Work	13

1.5 Discussion	16

1.6 Project Deliverables	17

1.7 Project Tasks	18

Reference	21

Table of Figures

Figure 1: Gantt Chart	4

Table of Tables

Table 1: Project Tasks Description	4

## 1.1 Background

The vast and inhospitable desert landscapes of the Middle East specifically the Rub' al Khali in Saudi Arabia, the deserts of the UAE, and the distinct terrain of Wadi Rum in Jordan pose a severe challenge to Search and Rescue (SAR) operations. Traditional rescue methods rely heavily on manned aviation or ground patrols, which are resource-intensive, slow to deploy, and limited by human fatigue. Crucially, these remote regions often lack cellular network coverage, rendering standard cloud-based drone solutions ineffective. This project addresses the critical need for an autonomous, offline SAR system capable of operating in "signal-denied" environments to locate lost individuals and vehicles with high precision and reduced latency.

Our proposed solution utilizes a specialized Unmanned Aerial Vehicle (UAV) equipped with an onboard Edge AI computing module. Unlike conventional drones that stream video to a pilot, this system functions as an autonomous scout. It executes a pre-programmed path planning algorithm to sweep designated search zones without human intervention. The core innovation lies in its "Edge Intelligence"; the drone runs a lightweight, high-speed object detection model (based on the YOLO architecture) locally on the device. This allows the system to analyze aerial imagery in real-time, identifying humans and vehicles instantly without needing an internet connection to process the data.

To ensure robustness in the unique topography of the region, the computer vision model is specifically trained to overcome local environmental anomalies. This includes "Domain Adaptation" techniques to handle the high-contrast glare of the UAE dunes and the specific spectral signature of the "Red Sands" in Wadi Rum, which often confuse standard detection algorithms. Furthermore, the system addresses the "Thermal Crossover" phenomenon where sand temperatures match human body heat by prioritizing RGB optical detection during critical daylight hours when thermal imaging is unreliable. This dual-focus approach ensures the system remains effective across the varied and extreme conditions of the Middle East.

The operational workflow is designed for maximum reliability: upon detecting a potential victim, the system logs the precise GPS coordinates and captures a timestamped image locally. Once the mission is complete or battery levels reach a critical threshold, the drone automatically returns to the launch point, allowing rescue teams to immediately access the coordinates of located targets. By automating the detection process and removing the dependency on live connectivity, this project offers a scalable, cost-effective, and life-saving tool that significantly reduces the "Time to Rescue" in some of the most unforgiving environments on Earth.

## 1.2 Motivation

The motivation for developing an autonomous, offline Search and Rescue (SAR) drone stems from a critical convergence of humanitarian necessity, economic efficiency, and the strategic expansion of tourism in the Middle East. As the region aggressively develops "Extreme Tourism" in its deserts (like: AlUla, Neom, Wadi Rum), the gap between the number of visitors and the capacity of traditional rescue infrastructure is widening.

### 1.2.1 The Humanitarian Crisis: The Sensitive Few Hours in the Empty Quarter

The primary motivation is the preservation of human life in environments where time is the only currency that matters. In the vast expanses of the Saudi desert, the margin for error is non-existent. Statistics from the Ingad Search and Rescue Association (Saudi Arabia) reveal a sobering reality: in a single year, over 142 people were reported missing in the desert, with 28 confirmed fatalities due to dehydration and heat exhaustion. Most of these deaths occurred because victims could not be located within the critical 60-minute window following an accident where medical intervention is most effective. In Jordan’s Wadi Rum, the challenge is compounded by flash floods and climbing accidents. Between 2018 and 2023, multiple incidents involving tourists and flash floods highlighted the inability of ground teams to traverse water-logged wadis or scale 1,500m cliffs rapidly. An autonomous drone does not wait for floodwater to recede; it flies over them, providing a lifeline when ground vehicles are paralyzed.

### 1.2.2 The Economic Driver: Cost-Asymmetry of Rescue

Traditional SAR operations are financially unsustainable for widespread coverage. A single hour of flight time for a rescue helicopter cost around $4,000 in fuel, crew, and maintenance. Furthermore, helicopters cannot fly safely in the severe sandstorms common to the Rub' al Khali due to engine intake risks. In stark contrast, the proposed drone system operates at a fraction of this cost (less than $5 USD per charge). This "Cost Asymmetry" allows for a new operational doctrine: instead of launching one expensive helicopter after a distress call is confirmed, Civil Defense forces can deploy "swarms" of cheap, autonomous drones to patrol high-risk areas proactively. This project democratizes safety, making advanced aerial SAR available to local volunteer groups (like Aoun and Ingad) who currently rely on private 4x4 vehicles.

### 1.2.3 Strategic Alignment: Supporting Vision 2030 & Regional Growth

Finally, this project aligns directly with the strategic vision of the Gulf Cooperation Council (GCC) nations. Saudi Arabia’s Vision 2030 aims to attract 150 million tourists annually, with a heavy emphasis on "Adventure Tourism" in remote desert locations like the Red Sea Project and the Tuwaiq Mountains. Similarly, Jordan’s economy relies heavily on the safety reputation of the Golden Triangle (Aqaba-Wadi Rum-Petra). Tourists will not visit remote deserts if they perceive them as death traps. By establishing a safety net of autonomous SAR drones, this project provides the "technological insurance" required to make these ambitious tourism goals viable. It transforms the desert from a terrifying void into a managed, safe destination, directly supporting the national economic agendas of Saudi Arabia, the UAE, and Jordan.

Current manual methods are too slow for the heat, too expensive for constant patrol, and too risky for human rescuers during sandstorms. This project is not merely an academic exercise; it is a necessary technological intervention to close the gap between the hostile reality of our geography and the value we place on human life.

## 1.3 Problem Statement

Search and Rescue (SAR) operations in desert and remote environments are time-critical and resource-intensive, often relying on manual ground searches or manned aerial surveys. These traditional approaches are slow, costly, and expose rescue teams to significant risks, while the vast terrain, harsh lighting conditions, sand-colored backgrounds, and low visual contrast make locating missing or injured individuals extremely challenging.

Recent advances in unmanned aerial vehicles (UAVs) and computer vision have shown promise in improving SAR efficiency. However, many existing AI-based detection systems are developed and evaluated under controlled or urban conditions and do not adequately address the unique visual and operational constraints of desert environments. Furthermore, most approaches focus solely on object detection accuracy, without considering system-level performance, robustness under degraded visual conditions, or suitability for real-time deployment on resource-constrained platforms.

Search and rescue (SAR) operations in remote, dangerous, or inaccessible environments often suffer from delays, limited human visibility, and inefficient manual scanning of large areas. Traditional SAR teams rely on ground searchers and helicopters, which are slow, costly, and sometimes unable to locate missing individuals quickly especially during natural disasters, at night, or in areas with obstructed terrain.

Therefore, there is a need for an end-to-end AI-based Search and Rescue Drone System that can reliably detect and track humans in desert environments in real-time while accounting for environmental challenges and practical deployment constraints. This project aims to design, implement, and evaluate such a system, focusing on robustness, efficiency, and real-world applicability to support faster and safer SAR operations.

## 1.3.1 Project Objectives

To address the technical gaps identified, the project aims to achieve the following:

- Deep Learning Pipeline Optimization: Develop a custom detection pipeline using a state-of-art Object Detection model Like Yolo. The target is to achieve a Mean Average Precision of 95% or more for human or vehicle detection. This high threshold is necessary due to the high stakes in SAR operations.

- Edge Hardware Integration: Successfully deploy and optimize these models on the Raspberry Pi 5. This involves leveraging hardware acceleration (like the Pi’s AI Hat) to ensure the model runs efficiently without overheating the unit.

- Top-Down Perspective Training: Unlike standard datasets that look at humans from the front, this project focuses on aerial perspectives, training the model to recognize shapes and heat signatures from a vertical or high-oblique angle.

- To evaluate the robustness of the system under degraded conditions such as low contrast, motion blur, and varying illumination.

- To provide a structured evaluation of detection performance, system reliability, and practical deployment limitations.

### Main Requirements

- The system shall assist search and rescue teams in locating missing or injured individuals in large desert areas using aerial surveillance.

- The system shall provide clear visual indications of detected humans to reduce manual monitoring effort by drone operators.

- The system shall operate effectively under challenging environmental conditions, including high sunlight intensity, low contrast backgrounds, and partial occlusions.

- The system shall support near real-time detection to enable timely decision-making during rescue missions.

- The system shall be easy to operate by rescue personnel without requiring advanced technical or AI expertise.

- The system shall reduce the time and human effort required to conduct search and rescue operations compared to traditional manual methods.

### 1.3.2 User Requirements

## The primary users of the proposed system are search and rescue (SAR) operators and emergency response teams conducting missions in desert and remote environments. The system must support their operational needs by providing timely, reliable, and actionable information while minimizing risk and resource usage.

### Functional Requirements

## Human Detection Capability

## The system shall be able to detect humans from aerial imagery captured by a UAV.

## Detection shall support top-down and high-oblique viewing angles typical of drone flights.

## The system shall provide bounding boxes and confidence scores for detected humans.

## Real-Time Operation

## The system shall perform inference in real time or near real time to support time-critical SAR missions.

## Detection results shall be generated continuously from a live or simulated drone video stream.

## Desert Environment Robustness

## The system shall operate reliably in desert-specific conditions, including:

## Low visual contrast between humans and background

## Harsh and uneven lighting

## Motion blur due to drone movement

## Partial occlusion and small target sizes

## Autonomous Processing

## The system shall operate with minimal user intervention once deployed.

## No manual image selection or annotation shall be required during inference.

## Visual Feedback for Operators

## The system shall display detection results visually, overlaying bounding boxes on the video feed.

## Confidence levels shall be presented in an interpretable manner to assist decision-making.

## Non-Functional Requirements

## Accuracy and Reliability

## The system shall achieve high detection accuracy, targeting a mean Average Precision (mAP) greater than 95% for relevant classes.

## False negatives shall be minimized, as missed detections directly impact mission success.

## Edge Deployment Compatibility

## The system shall be deployable on a Raspberry Pi 5 or equivalent edge device.

## The solution shall function within the computational and memory constraints of the platform.

## Performance Efficiency

## The system shall maintain acceptable inference speed (frames per second) suitable for live aerial monitoring.

## Resource usage (CPU, memory, and temperature) shall remain within safe operational limits.

## System Stability

## The system shall run continuously for extended periods without crashing or significant performance degradation.

## The system shall recover gracefully from temporary input interruptions (e.g., dropped frames).

## Portability and Ease of Deployment

## The system shall be easy to set up and deploy in simulated or real SAR environments.

## Configuration and execution shall be manageable by users with basic technical training.

### 1.3.4 Operational Constraints

- Operating under the limited processing power of edge hardware without relying on cloud connectivity.

- Extreme lighting conditions and visually homogeneous backgrounds common in desert terrains.

- The need to support near real-time inference without compromising mission duration or drone stability.

- Training and evaluation data may not fully represent real-world SAR conditions, requiring the system to generalize effectively to unseen scenarios.

- Communication between the drone and the ground station may be intermittent or bandwidth-limited, Drone system needs to be fully autonomous.

## 1.3.5 Relevant Computing Characteristics

The proposed AI-based Search and Rescue (SAR) Drone System exhibits several key computing characteristics that directly influence its design, implementation, and evaluation. These characteristics arise from the need to process visual data in real time, operate under resource constraints, and maintain reliability in challenging environmental conditions.

### Real-Time Processing

Search and rescue operations are inherently time-critical, requiring rapid analysis of incoming data to support immediate decision-making. The system must process continuous video streams from an aerial platform and perform object detection with minimal latency. This real-time requirement influences model selection, pipeline design, and optimization strategies, prioritizing low inference delay and stable frame rates over purely offline accuracy.

### Edge Computing Constraints

The system is designed for deploying on edge hardware, specifically the Raspberry Pi 5. Unlike cloud-based systems, edge devices have limited computational power, and memory. As a result, the system must employ lightweight deep learning architectures, optimized inference engines, and efficient memory management. Hardware acceleration are critical computing consideration to ensure sustained performance without overheating or system failure.

### High-Throughput Visual Data Processing

Drone-mounted cameras generate large volumes of high-resolution image data. The system must efficiently handle this continuous data stream, including frame acquisition, preprocessing, inference, and visualization. Efficient data pipelines and buffering mechanisms are required to prevent bottlenecks and dropped frames, especially during prolonged operation.

### Robustness to Noisy and Degraded Inputs

Desert environments introduce significant visual noise, including low contrast, motion blur, glare, and background homogeneity. From a computing perspective, the system must be robust to degraded input data without excessive computational overhead. This necessitates careful preprocessing, data augmentation during training, and model architectures capable of extracting discriminative features under challenging conditions.

### System Reliability and Fault Tolerance

The system must operate continuously in demanding conditions without frequent restarts or failures. From a computing perspective, this requires stable software execution, graceful handling of dropped frames or temporary sensor interruptions, and robust logging mechanisms. Reliability is particularly critical in SAR scenarios, where system downtime can directly impact mission outcomes.

## 1.4 Related Work

a literature review relevant to the topic, and evidence of research on existing solutions in computing.

The deployment of Unmanned Aerial Vehicles (UAVs) in Search and Rescue (SAR) operations represents one of the most significant technological advancements in modern humanitarian aid and disaster response. While the utility of UAVs has been extensively documented in urban, maritime, and forested environments, the desert ecosystem presents a distinct and formidable set of challenges that necessitates a specialized approach to computer vision and robotics. This report provides an exhaustive literature review and analysis of the current state of human detection technologies specifically tailored for arid, desert environments. It synthesizes findings from recent experimental benchmarks, algorithmic evolutions ranging from classical two-stage detectors like Faster R-CNN to state-of-the-art single-stage models like YOLOv8 and Transformer-based architectures and examines the results of major regional research initiatives such as the Mohamed Bin Zayed International Robotics Challenge (MBZIRC) and the "Shaheen" project at the American University of Sharjah.

The urgency of this research domain is underscored by the inherent lethality of the desert environment. Unlike temperate forests or urban zones where exposure risks may be mitigated by shelter or moderate temperatures, the desert is characterized by extreme heat, rapid dehydration risks, and vast, featureless topographies that disorient victims. In such scenarios, the time-to-detection is the single most critical variable determining survival. Consequently, the performance of detection algorithms is not merely an academic metric of Mean Average Precision (mAP) or Frames Per Second (FPS), but a direct correlate to mortality rates. The literature indicates that as time passes, the search area grows exponentially while the probability of survival decreases linearly, creating a critical window where automated, high-speed detection is mandatory.

Evaluation of Alternative Project Ideas

To ensure the selected capstone project represents the optimal solution for the identified problem domain, two alternative computing concepts were evaluated. These alternatives were assessed based on technical feasibility, data accessibility, and their suitability for the specific environmental constraints of the region.

Alternative Idea 1: Integrated AI Surveillance System for Critical Infrastructure

This concept proposed an intelligent software layer designed to upgrade existing CCTV networks in industrial facilities. The system aimed to utilize a multi-model computer vision architecture to autonomously detect specific hazard classes, including fire, smoke, water leakage, and unauthorized intrusion. The objective was to transform passive security feeds into a proactive real-time alerting system for high-risk indoor environments.

While the project was technically feasible using standard computer vision libraries, it was rejected due to its heavy reliance on established infrastructure. The system depends entirely on fixed cameras, power grids, and stable internet connectivity. Consequently, it fails to address safety requirements in off-grid or remote environments, such as the Jordanian Badia or Wadi Rum. Furthermore, the static nature of CCTV restricts the system to a fixed field of view, rendering it incapable of actively searching for missing persons in vast, unexplored terrains.

Alternative Idea 2: AI-Driven Urban Traffic Management System

This alternative focused on developing an adaptive traffic control system to alleviate congestion in metropolitan areas. The proposed solution involved using computer vision on live traffic feeds to calculate vehicle density at intersections and dynamically adjust signal timings using reinforcement learning, replacing traditional static timers.

Although traffic congestion is a significant local issue, this concept was rejected primarily due to data inaccessibility. Developing a functional model would require access to real-time government traffic feeds or historical density data, which is restricted and difficult to obtain. Additionally, simulating the system without physical implementation lacks the hardware engineering component required for a robust capstone project. Unlike a search and rescue system, which addresses immediate life-safety concerns, this alternative prioritizes convenience and efficiency.

Conclusion

Following a comparative evaluation, the Autonomous Search and Rescue Drone was selected as the superior project. Unlike the surveillance system, the drone operates independently of external infrastructure, carrying its own power and processing units to function in remote desert regions. In contrast to the traffic management system, the drone addresses a critical humanitarian need by aiding in the recovery of lost individuals in harsh environments. This choice offers a balanced integration of software and hardware engineering, fulfilling the comprehensive requirements of the computing capstone.

## 1.5 Discussion

To ensure the project focuses on high-level computing objectives, the system is designed as a Modular Intelligent Payload. This payload is platform-agnostic, meaning it can be mounted onto any "Ready-to-Fly" UAV (Unmanned Aerial Vehicle) capable of carrying a 500g payload. The assessment below focuses strictly on the computing, vision, and software integration components.

1. Hardware Components (The Intelligent Payload)

The system consists of a self-contained processing unit designed to attach to the carrier drone.

- Primary Computing Unit: Raspberry Pi 5 (8GB RAM). Selected for its quad-core Cortex-A76 processor and PCIe interface. It serves as the mission computer, handling operating system tasks, telemetry logging, and communication with the flight controller.

- AI Acceleration Module: Raspberry Pi AI Kit (Hailo-8L). This M.2 HAT+ module provides 13 TOPS of neural processing power. It is dedicated exclusively to running the computer vision models, ensuring the main CPU remains free for critical system operations.

- Vision Sensor: Raspberry Pi Camera Module 3 (Wide). This camera connects via the CSI-2 interface. The "Wide" variant is selected to maximize the field of view (FOV) during search operations, allowing the system to scan larger swaths of terrain at lower altitudes.

- Interface to Carrier Drone: A UART-to-USB connection links the Raspberry Pi to the carrier drone’s Flight Controller This allows the payload to read GPS data and send high-level navigation without needing to manage low-level motor control.

2. Software Architecture

The software is designed as a three-tier stack running on the Raspberry Pi:

- Tier 1: Perception Layer

  - Model: YOLOv8.

  - Engine: AI Hat (Runtime).

  - Function: Processes raw video frames at 60 FPS to identify human targets.

- Tier 2: Logic Layer

  - Language: Python 3.11.

  - Logic: A state machine that filters detection noise. It requires a target to be detected in at least 5 consecutive frames before declaring a "Confirmed Find," preventing false alarms from shadows or rocks.

- Tier 3: Control Layer

  - Library: DroneKit-Python

  - Function: Translates the "Confirmed Find" signal into Control Layer commands sent to the carrier drone to halt movement and tag the location.

3. System Interfaces

- Physical Interface: Custom 3D-printed mount (PLA) designed to house the Pi 5, AI HAT, and Camera, protecting them from dust and vibration while ensuring proper airflow for cooling.

- Operator Interface: A web-based dashboard hosted on the Raspberry Pi The operator connects via Wi-Fi to view the live video stream and detection status during pre-flight checks or close-range operations.

## 1.6 Project Deliverables

Upon the conclusion of the Capstone project, the following artifacts will be delivered:

- Final Technical Report: A formal academic document detailing the complete project lifecycle. This report includes the problem definition, literature review, detailed system architecture (hardware and software specifications), implementation methodology, and a quantitative evaluation of the system's performance.

- Project Presentation: A formal presentation summarizing the research, design, and implementation phases of the project. This includes a technical slide deck and an oral defense of the engineering decisions made during the development process.

- Integrated Hardware Prototype: A functional "Intelligent Search Payload" consisting of the embedded computing unit (Raspberry Pi 5), the neural processing accelerator (RPI AI HAT), and the optical sensor array. The components will be integrated into a custom-fabricated enclosure compatible with standard UAV airframes.

- Software Source Code: The complete software repository required to replicate the system functionality. This includes:

  - Trained Model

  - Python Computer Vision Pipeline

  - All Utilized Data for Training and Validation

- System Validation (Proof of Concept): Documented evidence of the system's operational capability. This includes test logs and video documentation demonstrating the successful execution of autonomous target detection and telemetry override in a controlled test environment.

## 1.7 Project Tasks

Develop a detailed project timeline using project management tools (e.g., project schedule).

Figure 1: Gantt Chart

Table 1: Project Tasks Description

| Phase 1: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Task ID | Task Name | Description | Time Duration | Assigned To | Notes |
| Phase 2: |  |  |  |  |  |

# Reference

- HUMAN DETECTION FROM AERIAL IMAGERY FOR FOR SEARCH AND RESCUE OPERATIONS - ResearchGate, https://www.researchgate.net/publication/378371948_HUMAN_DETECTION_FROM_AERIAL_IMAGERY_FOR_FOR_SEARCH_AND_RESCUE_OPERATIONS

- AUS Students' Shaheen Project Earns Global Recognition for Desert Rescue Innovation, , https://www.education-uae.com/aus-students-shaheen-desert-rescue-project-earns-global-recognition/

- News | Page 2 | American University of Sharjah, https://www.aus.edu/media/news/2025?field_news_label_tid=All&og_group_ref_target_id_entityreference_filter=41&page=1

- Bringing Intelligence to SAR Missions: A Comprehensive Dataset and Evaluation of YOLO for Human Detection in TIR Images - IEEE Xplore, https://ieeexplore.ieee.org/iel8/6287639/10820123/10840181.pdf

- Real-Time Person Detection in Wooded Areas Using Thermal Images from an Aerial Perspective - NIH, https://pmc.ncbi.nlm.nih.gov/articles/PMC10675173/

- DCS-YOLOv8: A Lightweight Context-Aware Network for Small Object Detection in UAV Remote Sensing Imagery - MDPI, https://www.mdpi.com/2072-4292/17/17/2989

- UAV multispectral multi-domain feature optimization for the air-to-ground recognition of outdoor injured human targets under cross-scene environment - PubMed Central, https://pmc.ncbi.nlm.nih.gov/articles/PMC9947796/

- Thermal human detection for Search & Rescue UAVs - DiVA portal, https://www.diva-portal.org/smash/get/diva2:1707781/FULLTEXT01.pdf

- Benchmarking YOLOv5 models for improved human detection in search and rescue missions - ResearchGate, https://www.researchgate.net/publication/379023256_Benchmarking_YOLOv5_models_for_improved_human_detection_in_search_and_rescue_missions

- Target objects at the desert location | Download Scientific Diagram - ResearchGate, https://www.researchgate.net/figure/Target-objects-at-the-desert-location_fig5_261261564

- Archangel: A Hybrid UAV-based Human Detection  - arXiv, https://arxiv.org/pdf/2209.00128

- Artificial intelligence takes to the skies to protect a Saudi tradition - KAUST, https://www.kaust.edu.sa/news/artificial-intelligence-takes-to-the-skies-to-protect-a-saudi-tradition

- Small Object Detection in UAV Remote Sensing Images Based on Intra-Group Multi-Scale Fusion Attention and Adaptive Weighted Feature Fusion Mechanism - MDPI, https://www.mdpi.com/2072-4292/16/22/4265

- YOLO Vs Faster RCNN for Object Detection and Recognition - International Research Journal of Multidisciplinary Scope (IRJMS), https://www.irjms.com/wp-content/uploads/2025/07/Manuscript_IRJMS_04661_WS.pdf

- Drone-Assisted Plant Stress Detection Using Deep Learning: A Comparative Study of YOLOv8, RetinaNet, and Faster R-CNN - MDPI, https://www.mdpi.com/2624-7402/7/8/257

- Agricultural Greenhouses Detection in High-Resolution Satellite Images Based on Convolutional Neural Networks: Comparison of Faster R-CNN, YOLO v3 and SSD - PMC - NIH, https://pmc.ncbi.nlm.nih.gov/articles/PMC7506698/

- Comparative Analysis of YOLO and Faster R-CNN Models for Detecting Traffic Object - The Science and Information (SAI) Organization, https://thesai.org/Downloads/Volume16No3/Paper_42-Comparative_Analysis_of_YOLO_and_Faster_R_CNN_Models.pdf

- UAV-YOLOv8: A Small-Object-Detection Model Based on Improved YOLOv8 for UAV Aerial Photography Scenarios - PMC - NIH, https://pmc.ncbi.nlm.nih.gov/articles/PMC10458807/

- HUMAN DETECTION IN SEARCH AND RESCUE OPERATIONS USING EMBEDDED ARTIFICIAL INTELLIGENCE | Jurnal Teknologi (Sciences & Engineering), https://journals.utm.my/jurnalteknologi/article/view/19497

- Synergistic fusion: An integrated pipeline of CLAHE, YOLO models, and advanced super-resolution for enhanced thermal eye detection - PubMed Central, https://pmc.ncbi.nlm.nih.gov/articles/PMC12273955/

- Thermal Infrared-Image-Enhancement Algorithm Based on Multi-Scale Guided Filtering, https://www.mdpi.com/2571-6255/7/6/192

- Enhanced Thermal Human Detection with Fast Filtering for UAV, https://www.semanticscholar.org/paper/Enhanced-Thermal-Human-Detection-with-Fast-for-UAV-Ozyurt-Cicekdag/669a70722740e56d35416f285f5e150a2bad5421

- Adaptive Contrast Enhancement for Infrared Images Based on the Neighborhood Conditional Histogram - MDPI, https://www.mdpi.com/2072-4292/11/11/1381

- Transformer-Based Person Detection in Paired RGB-T Aerial Images With VTSaR Dataset - IEEE Xplore, https://ieeexplore.ieee.org/iel8/4609443/10766875/10833840.pdf

- Detecting Humans in Search and Rescue Operations Based on Ensemble Learning, https://ieeexplore.ieee.org/document/9729188/

- Semi-Supervised Person Detection in Aerial Images with Instance Segmentation and Maximum Mean Discrepancy Distance - MDPI, https://www.mdpi.com/2072-4292/15/11/2928

- SARD - Search And Rescue Dataset - Kaggle, https://www.kaggle.com/datasets/nikolasgegenava/sard-search-and-rescue

- SARD 2 - Search And Rescue Dataset, Extra Classes - Kaggle, https://www.kaggle.com/datasets/nikolasgegenava/sard-2-search-and-rescue-dataset-extra-classes/data

- AUS students' desert rescue project earns global recognition | American University of Sharjah, https://www.aus.edu/media/news/aus-students-desert-rescue-project-earns-global-recognition

- Caltech Aerial RGB-Thermal Dataset in the Wild - CaltechDATA, https://data.caltech.edu/records/cks6g-ps927

- aerorobotics/caltech-aerial-rgbt-dataset - GitHub, https://github.com/aerorobotics/caltech-aerial-rgbt-dataset

- Small Object Detection in UAV Images Based on YOLOv8n - ResearchGate, https://www.researchgate.net/publication/383449698_Small_Object_Detection_in_UAV_Images_Based_on_YOLOv8n

- Team JSK at MBZIRC 2020: Interception of fast flying target using multilinked aerial robot - IEEE Xplore, https://ieeexplore.ieee.org/iel8/10854677/10878395/10878398.pdf

- Finalist Teams | MBZIRC, https://www.mbzirc.com/finalist-teams

- Grand Challenge | MBZIRC, https://www.mbzirc.com/grand-challenge

- (PDF) A Multi-MAV System for the Autonomous Elimination of Multiple Targets in the MBZIRC 2020 Competition - ResearchGate, https://www.researchgate.net/publication/362512743_A_Multi-MAV_System_for_the_Autonomous_Elimination_of_Multiple_Targets_in_the_MBZIRC_2020_Competition

- A Benchmark and Simulator for UAV Tracking (Dataset), https://ivul.kaust.edu.sa/benchmark-and-simulator-uav-tracking-dataset

- Human Detection in UAV Thermal Imagery: Dataset Extension and Comparative Evaluation on Embedded Platforms - MDPI, https://www.mdpi.com/2313-433X/11/12/436

- Synergistic fusion: An integrated pipeline of CLAHE, YOLO models, and advanced super-resolution for enhanced thermal eye detection - EconPapers, https://econpapers.repec.org/article/plopone00/0328227.htm

- Human Detection in UAV Thermal Imagery: Dataset Extension and Comparative Evaluation on Embedded Platforms - NIH, https://pmc.ncbi.nlm.nih.gov/articles/PMC12733663/

- Real-Time Search and Rescue with Drones: A Deep Learning Approach for Small-Object Detection Based on YOLO - MDPI, https://www.mdpi.com/2504-446X/9/8/514

- Reducing Object Detection Uncertainty from RGB and Thermal Data for UAV Outdoor Surveillance - CSIRO Research Publications Repository, https://publications.csiro.au/publications/#publication/PIcsiro:EP2021-2869/RI17/RT39

- Non-terrestrial Communication System for Aerial-based Object Recognition and Positioning, https://admissions.kaust.edu.sa/internship/search/non-terrestrial-communication-system-for-aerial-based-object-recognition-and-positioning

- Aviation Solutions. (2025). How Much Does Helicopter Fuel Cost Per Hour? Available at: https://aviationsolutions.co/helicopter-fuel-cost-per-hour.

- https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=10716363

- https://arxiv.org/pdf/2209.00128

‌

Learning Outcomes and Assessment Criteria

Capstone Semester 1

| Pass | Merit | Distinction |
| --- | --- | --- |
| LO1 Demonstrate an ability to produce a well-planned computing project concept |  | D1 Evaluate alternative project ideas and assess their feasibility and suitability based on computing considerations. |
| P1 Conduct a comprehensive background study, including the importance of solving the problem, a literature review relevant to the topic, and evidence of research on existing solutions in computing. P2 Describe the problem in detail, including user requirements, objectives, constraints, and relevant computing characteristics. P3 Develop a detailed project timeline using project management tools (e.g., project schedule). | M1 Assess the selected idea in terms of system components, functionalities, and interfaces. M2 Write and present a professional project proposal. |  |